SGS HAJJ LUGGAGE PLATFORM

Document 2: SGS BagScan — Dedicated Native Scanning App

Technical Specification & Scope — React Native · Expo · Zebra SDK · Offline-First

Semicolon Technology Company  |  April 2026  |  Confidential

# 1. Executive Summary

This document specifies a dedicated native scanning application — SGS BagScan — to replace the camera-based scanning module currently embedded in the SGS Hajj Luggage web platform. The app targets two hardware profiles simultaneously: consumer Android/iOS smartphones used by some airport agents, and Zebra enterprise handhelds (TC57HO, TC72, TC77, MC93) used by SGS ground operations teams.

The PWA embedded in the web platform has fundamental architectural limitations that cannot be fully resolved without a dedicated app: browser security restrictions on camera access, the inability to use Zebra's DataWedge SDK, battery drain from continuous camera scanning, and poor performance in noisy airport lighting conditions. A native app eliminates all of these in one step.

| Category | PWA (Current) | SGS BagScan (Proposed) |
| --- | --- | --- |
| Scan speed (camera) | ~150ms decode + 1–2s latency | N/A — hardware scanner used on Zebra |
| Scan speed (hardware) | Not supported | < 80ms (Zebra DataWedge laser) |
| Zebra SDK access | No | Full DataWedge SDK via Expo module |
| Offline capability | Partial (scan queue only) | Full offline scan + local manifest |
| Background sync | Browser-limited | Native background fetch |
| Battery impact | High (camera active) | Low (hardware scanner, no camera needed) |
| Installation | Add to Home Screen (browser) | APK / Play Store / MDM push |
| Update mechanism | Auto on next load | OTA update via Expo + forced update path |

# 2. Technology Stack Decision

## 2.1 Recommended: React Native with Expo

React Native with Expo is the correct choice for this project for three specific reasons aligned with the existing codebase:

- Shared business logic. The existing SGS platform is React + TypeScript. The API client, data models, offline queue logic, and barcode validation utilities can be shared directly via a monorepo package. Zero duplication of business rules.

- Zebra DataWedge support. The community package @hapi/zebra-datawedge-react-native provides a stable bridge to Zebra's DataWedge intent system on Android, which is the official Zebra integration path. Expo's bare workflow allows installing this as a native module.

- Single codebase, two platforms. One React Native codebase targets Android (for Zebra devices — all Zebra handhelds run Android) and iOS (for agent smartphones). Expo's managed workflow handles build configuration for both.

## 2.2 Project structure

sgs-platform/               ← existing web platform (stays as is)

client/

server/

shared/

sgs-bagscan/                ← new native app (separate repo or monorepo package)

src/

screens/

hooks/

lib/

android/                  ← Zebra-specific native config

ios/

sgs-shared/                 ← shared package

api.ts                    ← typed API client

barcodeUtils.ts           ← GS1 stripping, validation regex

offlineQueue.ts           ← scan event queue (shared logic)

types.ts                  ← Bag, Flight, FlightGroup types

## 2.3 Key dependencies

| Package | Purpose | Notes |
| --- | --- | --- |
| expo ~51 | Build system, OTA updates | Use bare workflow for DataWedge native module |
| react-native ~0.74 | Core framework |  |
| @zebra-technologies/datawedge | Zebra hardware scanner integration | Official Zebra DataWedge intent bridge. Auto-detects DataWedge availability. |
| vision-camera v4 + MLKit plugin | Camera scanning for non-Zebra devices | 10× faster than html5-qrcode. CODE_128 native decode in < 50ms. |
| @react-native-async-storage | Lightweight key-value store | For auth token, session state, user preferences |
| WatermelonDB | Offline-first local database | SQLite-backed, reactive queries. Replaces IndexedDB for the manifest cache. |
| React Navigation v6 | Navigation stack | Stack + Tab navigators for the 3-screen flow |
| @tanstack/react-query | Server state + caching | Same library as web platform — same patterns |
| expo-updates | OTA code updates | Push fixes without Play Store/APK deployment |
| expo-background-fetch | Background sync | Flush offline queue when device comes online |

# 3. Zebra DataWedge Integration

## 3.1 What DataWedge is

DataWedge is Zebra's pre-installed middleware on all enterprise handhelds (TC57HO, TC72, TC77, MC93, PS20, etc.). It runs as a background service and manages the hardware scanner. Applications communicate with it via Android Intents. DataWedge handles all barcode decoding at the hardware/firmware level — the app only receives the decoded string via a broadcast intent.

This is fundamentally different from any camera-based approach. The hardware laser or imager reads the barcode, the Zebra firmware decodes it in < 80ms, and DataWedge broadcasts the result. The React Native app receives a broadcast intent and processes the string. No camera permissions, no frame analysis, no GPU usage.

## 3.2 DataWedge profile configuration

Create a DataWedge profile named SGS-BagScan that is automatically activated when the app is in the foreground:

Profile: SGS-BagScan

Associated app:  com.semicolon.sgsbagscan

Activity:        *  (all activities)

Barcode input:   ENABLED

Symbologies:     Code 128 ONLY  (disable all others)

Intent output:   ENABLED

Intent action:   com.semicolon.sgsbagscan.SCAN

Intent delivery: Broadcast

String tag:      com.symbol.datawedge.data_string

Keystroke output: DISABLED  (use Intent, not keyboard wedge)

This profile can be deployed to all Zebra devices via Zebra StageNow or an MDM (MobileIron, SOTI, etc.) so agents do not need to configure anything manually.

## 3.3 React Native DataWedge receiver

import { DeviceEventEmitter } from 'react-native';

import DataWedge from '@zebra-technologies/datawedge-react-native';

export function useZebraScanner(onScan: (tag: string) => void) {

useEffect(() => {

// Send intent to DataWedge to register our profile

DataWedge.sendCommand('com.symbol.datawedge.api.SWITCH_TO_PROFILE', 'SGS-BagScan');

const sub = DeviceEventEmitter.addListener(

'datawedge_broadcast_intent',

(event) => {

const raw = event['com.symbol.datawedge.data_string'];

if (!raw) return;

const cleaned = stripGS1Controls(raw).toUpperCase();

if (isValidBagTag(cleaned)) onScan(cleaned);

}

);

return () => sub.remove();

}, [onScan]);

}

## 3.4 Fallback for non-Zebra devices

On iOS or non-Zebra Android, the app falls back to react-native-vision-camera with the vision-camera-code-scanner MLKit plugin. This provides native frame processing at 60fps with CODE_128 decode in under 50ms — approximately 3× faster than html5-qrcode in the browser due to native GPU access. The same onScan callback is used, so the scan processing logic is identical on both paths.

// Automatic hardware detection

const isZebra = await DeviceInfo.getManufacturer() === 'Zebra Technologies';

// In the scan screen:

{isZebra

? <ZebraScanView onScan={processScan} />          // DataWedge intent

: <VisionCameraScanView onScan={processScan} />  // MLKit native camera

}

# 4. App Screens & User Flows

## 4.1 Screen architecture (3 main screens)

The app has exactly 3 screens. This is intentional — airport belt agents need zero cognitive load. The workflow is linear: authenticate → select context → scan bags continuously.

| Screen | Name | Purpose |
| --- | --- | --- |
| Screen 1 | Login | JWT auth against the SGS platform API. Token stored in SecureStore. |
| Screen 2 | Session Setup | Select flight + group. Pre-cache manifest. Shows agent's assigned flights first. |
| Screen 3 | Scan | The main screen. Full-screen viewfinder or Zebra trigger mode. Green/Red/Yellow flash. Live counter. Offline indicator. |

## 4.2 Screen 1 — Login

- Full-screen SGS branding (black background, SGS green sunburst logo)

- Username + password fields, Touch ID / Face ID biometric unlock after first login

- JWT stored in expo-secure-store (hardware-backed keystore on Zebra)

- Token refresh runs silently in background — agent is never logged out mid-shift

- If server unreachable on login, show clear message: 'Connect to SGS network first'

- App version and last sync timestamp shown at bottom

## 4.3 Screen 2 — Session Setup

- Shows only flights assigned to this agent's station (stationCode from JWT)

- If duty_manager has assigned specific flights to this agent via flight_user_assignments, those appear at the top with 'ASSIGNED' badge

- Flight list cached locally — visible even offline (from previous session)

- Selecting a flight triggers immediate background manifest pre-fetch

- Manifest fetch progress shown: 'Loading manifest: 47 bags' with spinner

- Group selection shows accommodation name, expected bag count, and current status

- If manifest already cached from < 8 hours ago, shows 'Manifest ready (47 bags)' with green indicator

- 'START SCANNING' button becomes active once flight + group selected

## 4.4 Screen 3 — Scan (the core experience)

### Layout on Zebra device (hardware scanner mode)

- Full-screen black background

- Large SGS logo centered at top

- 'READY TO SCAN' in SGS green — large, clearly visible from arm's length

- Last scanned tag displayed in large monospace font

- Live counter: 'Bags scanned this session: 12 / 47 expected'

- Progress ring around the counter showing % of expected count

- Offline indicator at bottom if no connectivity

- Zebra trigger → flash overlay → counter increments → ready immediately

### Layout on phone (camera mode)

- Full-screen viewfinder (rear camera, rear-facing)

- Green bracket overlay on viewfinder indicating the scan zone

- Narrow horizontal target zone (optimized for 1D Code 128 barcodes)

- Live counter overlay at bottom of viewfinder

- Camera never turns off — continuous scanning

### Scan result flash overlay (both modes)

After every scan, a full-screen colored overlay appears for 800–1500ms, then disappears automatically. The agent never taps anything between scans.

| Color | State | Duration | Details shown |
| --- | --- | --- | --- |
| GREEN | In manifest, saved | 800ms | Bag tag in large text. 'COLLECTED' label. |
| RED | Not in manifest | 1500ms + 3 vibrates | 'NOT IN MANIFEST' label. Tag shown. Option to raise exception. |
| YELLOW | Queued offline | 1000ms | 'SAVED OFFLINE — will sync when connected' |
| AMBER border | Duplicate scan | 1000ms | 'ALREADY SCANNED' label. Original scan time shown. |
| ORANGE | Non-Hajj bag | 1500ms | 'OUT OF SCOPE' label. Bag exists but not a Hajj pilgrim. |

### Action buttons (available during scan session)

Shown in a bottom action bar — accessible with one thumb, no scrolling:

- EXCEPTION — raises exception for last scanned or manually entered tag

- NO TAG — opens no-tag surrogate flow (quick 2-tap: create surrogate → back to scanning)

- SYNC NOW — manual trigger to flush offline queue (auto-triggers when online)

- SESSION — change flight or group (warns if pending unsynced scans)

# 5. API Integration with the SGS Platform

## 5.1 The app is a pure API client

SGS BagScan does not contain any business logic that is not already in the platform API. It is a thin client that authenticates, fetches data, and posts events. This is a critical architectural constraint — all validation, bag state machine transitions, Nusuk push triggers, and group management happen on the server. The app only needs to surface the results.

## 5.2 API endpoints used by the app

| Endpoint | Method | Used For |
| --- | --- | --- |
| POST /api/auth/login | POST | Initial login. Returns JWT + sets refresh cookie. |
| POST /api/auth/refresh | POST | Silent token refresh. Called on app foreground. |
| GET /api/flights | GET | Session Setup screen: list of flights for the agent's station. |
| GET /api/flights/assignments-all | GET | Flights specifically assigned to this agent by duty manager (shown first). |
| GET /api/flight-groups?flightId=:id | GET | Group selection for chosen flight. |
| GET /api/flights/:id/bags | GET | Pre-cache manifest: fetch all bag tags for the flight. |
| POST /api/bags/:bagTag/events | POST | Core scan action. Posts COLLECTED_FROM_BELT or EXCEPTION event. |
| POST /api/bags/no-tag | POST | Create a surrogate bag for a tag-less physical bag. |
| POST /api/exceptions/raise | POST | Raise MISSING, DAMAGED, DELAYED, CUSTOMS_HOLD exception. |
| POST /api/flight-groups/:id/bulk-receive | POST | Bulk receive for a group (physical count entry). |

## 5.3 Authentication strategy

- JWT stored in expo-secure-store (hardware-backed on Zebra and iOS Secure Enclave)

- Refresh token stored alongside JWT — silent refresh on app foreground event

- If refresh fails (e.g., server unreachable), app continues offline with the cached token for up to 8 hours. After 8 hours the agent must re-authenticate.

- Biometric unlock (Touch ID / Zebra fingerprint reader) re-opens the app without requiring re-entry of username/password — only the initial login requires credentials

## 5.4 Offline queue and sync

The offline queue uses WatermelonDB (SQLite-backed) for durability. Unlike IndexedDB in the browser, SQLite on mobile survives app process kills and OS memory pressure. Every queued scan event is persisted to disk immediately.

// WatermelonDB schema for offline queue

tableSchema({

name: 'queued_scans',

columns: [

{ name: 'bag_tag',         type: 'string' },

{ name: 'event_type',      type: 'string' },

{ name: 'flight_id',       type: 'string' },

{ name: 'flight_group_id', type: 'string' },

{ name: 'queued_at',       type: 'number' },

{ name: 'retry_count',     type: 'number' },

{ name: 'max_retries',     type: 'number' },  // default 5

{ name: 'last_error',      type: 'string', isOptional: true },

]

})

The sync process uses expo-background-fetch to attempt sync even when the app is backgrounded. The agent puts the device in their pocket and walks to a WiFi area — sync completes automatically without re-opening the app.

Retry cap: max 5 retries with exponential backoff (2s, 4s, 8s, 16s, 32s). After 5 failures, the event is marked as FAILED and surfaced to the agent on next app open with a clear error message and the option to retry manually or discard.

# 6. Offline Manifest — Local Bag Tag Database

## 6.1 Architecture

When the agent selects a flight, the app fetches all bag tags (from GET /api/flights/:id/bags) and stores them in a local WatermelonDB table. This gives the agent a local copy of the manifest that is used for instant offline verification — no server round-trip needed.

## 6.2 Verification logic during scan

The scan decision tree is executed in < 5ms locally:

- Check session duplicate set (in-memory Set). If found → AMBER flash.

- Check local manifest cache. If tag found → GREEN flash (online: also post event; offline: queue event).

- If tag NOT found in cache and offline → RED flash 'Not in manifest (offline)'.

- If tag NOT found in cache and online → POST to API. If 200 → GREEN. If 404 → RED.

- If isHajjBag === false (once the platform refactor is deployed) → ORANGE flash 'Out of scope'.

## 6.3 Cache freshness

The manifest is re-fetched every time the agent opens a session for a flight. If the manifest has grown since the last cache (e.g., airline pushed additional passenger records), the cache is updated automatically. The cache is keyed by flightId and stores the timestamp of the last fetch. A stale indicator appears if the cache is more than 4 hours old while the device is online.

# 7. UX Design Principles for Airport Operations

## 7.1 Design constraints

Airport belt areas have specific conditions that are completely different from a standard mobile app:

- Bright sunlight or alternating dim/bright lighting (screen contrast matters)

- Agent is standing and moving — not holding the phone stationary

- Both hands may be occupied — app must work with one thumb

- Noise levels prevent audio feedback — visual and haptic only

- Agent scans 30–100 bags in 20–40 minutes — every second of friction matters

- Gloves may be worn — targets must be large (minimum 48dp touch area)

## 7.2 Design rules for SGS BagScan

- Minimum font size for operational data: 24sp. Scan results: 40sp+

- No dialogs or confirmations during normal scan flow — only for destructive actions

- Every result auto-dismisses. Agent never taps 'OK' or 'Close'

- Color feedback uses full-screen flash, not small toasts or banners

- The counter is always visible — 'bags scanned / bags expected' is the primary KPI

- Dark mode only — consistent with SGS brand, better contrast outdoors

- Screen timeout should be extended to 10 minutes during active scan session (expo-keep-awake)

- All buttons reachable with right thumb in portrait mode — no two-handed gestures

## 7.3 Haptic feedback profile

| Event | Pattern | Notes |
| --- | --- | --- |
| Successful scan | Single 50ms pulse | Short, clean — confirms decode |
| Not in manifest | Three 100ms pulses, 50ms gaps | Distinct error pattern |
| Duplicate scan | Two 75ms pulses | Different from error — not urgent |
| Session full / expected count reached | Long 300ms pulse | Milestone notification |
| Sync complete | None | Silent — do not interrupt scanning |

# 8. Zebra Enterprise Deployment

## 8.1 Distribution path

For Zebra enterprise devices managed by SGS IT, the app is distributed as an APK through the existing MDM solution (or Zebra StageNow if no MDM exists). This means:

- Agent does not visit the Play Store — IT deploys the APK silently

- DataWedge profile is pushed alongside the APK via StageNow profile

- Updates are delivered via expo-updates OTA — no new APK needed for code changes

- A new APK is only required when native modules change (rare)

## 8.2 Expo OTA update strategy

expo-updates allows pushing JavaScript bundle updates without going through APK deployment. Configure mandatory update enforcement for security-critical changes:

// app.config.js

updates: {

url: 'https://u.expo.dev/[project-id]',

fallbackToCacheTimeout: 0,

checkAutomatically: 'ON_LOAD',

},

// In app startup:

const update = await Updates.checkForUpdateAsync();

if (update.isAvailable) {

await Updates.fetchUpdateAsync();

// For mandatory updates, force reload immediately

// For optional updates, notify on next session

await Updates.reloadAsync();

}

## 8.3 Zebra-specific app configuration

- Request VIBRATE permission in AndroidManifest for haptic feedback

- Set android:screenOrientation to portrait (Zebra handheld use is always portrait)

- Configure DataWedge intent filter in AndroidManifest to receive scan broadcasts

- Set android:keepScreenOn during active scan session (battery trade-off accepted for operational reliability)

- Request FOREGROUND_SERVICE permission for background sync

# 9. Key API Contracts

## 9.1 POST /api/bags/:bagTag/events — primary scan action

Every successful scan posts this request body:

{

eventType:     'COLLECTED_FROM_BELT',  // always for airport agent

flightGroupId: '<uuid>',

flightId:      '<uuid>',

correlationId: '<session-uuid>',       // groups all scans in one session

locationName:  'JED Arrival Belt 3',   // static per session context

notes:         null

}

Expected responses the app must handle:

| HTTP | Meaning | App response |
| --- | --- | --- |
| 201 | Event recorded | GREEN flash, increment counter, continue scanning |
| 404 | Bag tag not found | RED flash 'Not in manifest'. Option to raise exception. |
| 422 | Non-Hajj bag (out of scope) | ORANGE flash 'Out of scope'. Log locally, do not re-try. |
| 409 | Status conflict (already past this state) | AMBER flash 'Already processed'. Show current bag status. |
| 401 | Auth expired | Attempt silent refresh. If fails, redirect to login. |
| Network error | Offline / timeout | YELLOW flash. Queue to offline store. Continue scanning. |

## 9.2 Offline event queue payload stored locally

{

bagTag:        'SV-JED-260510-013',

eventType:     'COLLECTED_FROM_BELT',

flightId:      '<uuid>',

flightGroupId: '<uuid>',

correlationId: '<session-uuid>',

queuedAt:       1718000000000,

retryCount:     0,

maxRetries:     5,

lastError:      null

}

# 10. Delivery Scope & Phase Plan

## Phase 1 — Core Scan App (4–5 weeks)

Delivers the minimum viable app for production use on Zebra devices at JED.

| Deliverable | Effort | Notes |
| --- | --- | --- |
| React Native + Expo project scaffolding, Zebra detection, DataWedge integration | 1 week | Core architecture |
| Login screen + JWT + SecureStore | 3 days | Reuse auth logic from web platform |
| Session Setup screen (flight + group selection) | 4 days | API client + offline list cache |
| Manifest pre-fetch + WatermelonDB local cache | 3 days | Core offline capability |
| Scan screen: Zebra hardware mode + visual flash feedback | 1 week | Primary user experience |
| Scan screen: camera mode (VisionCamera + MLKit) for non-Zebra | 4 days | Fallback for phones |
| Offline scan queue + background sync + retry with cap | 3 days | SQLite-backed queue |
| Exception raise flow | 2 days | Simple 2-screen flow |
| No-tag surrogate creation | 1 day | Calls existing API |
| APK build + DataWedge profile + StageNow deployment | 2 days | IT deployment |

## Phase 2 — Enhancements (3 weeks, after Phase 1 live)

- Biometric unlock (Touch ID / Zebra fingerprint)

- Bulk receive flow (physical count entry for group)

- Shift summary screen (end of shift: bags collected, exceptions raised, sync status)

- iOS build + App Store / enterprise distribution for agent smartphones

- Expo OTA update pipeline configuration

- Arabic language support (RTL layout, Arabic field labels)

## Phase 3 — Advanced (future)

- Push notifications to agent when duty manager assigns new flight

- Agent-to-agent real-time bag count sync (WebSocket via existing SSE endpoint)

- Photo capture for damaged bag exceptions

- RFID support on Zebra devices equipped with RFID reader (MC3300R)

# 11. Security Considerations

- JWT stored in hardware-backed secure storage (expo-secure-store) — never in AsyncStorage or SQLite

- API calls always use HTTPS — no HTTP fallback

- Offline queue contains bag tags and event types only — no passenger PII is stored locally

- The local manifest cache contains bag tags only (not passport numbers or pilgrim names) — if the device is lost, no PII is exposed

- The app should implement a 30-minute inactivity lock that requires biometric re-authentication

- Log all scan events locally with timestamp before posting to API — provides audit trail even if API fails

- App version is enforced via the OTA update system — outdated versions below a configured minimum are blocked from logging in

# 12. Success Metrics

| Metric | Target | Current PWA baseline |
| --- | --- | --- |
| Average time from pull trigger to visual confirmation | < 200ms (Zebra mode) | ~2 seconds (camera close + toast) |
| False decodes per 100 scans (GS1 garbage) | 0 | ~3–5 observed in field |
| Offline scan accuracy (no connectivity) | 100% — every scan queued correctly | Partial — no manifest verification |
| Agent taps required between scans | 0 | 1 (re-tap camera button) |
| Battery drain per 4-hour shift | < 15% (Zebra mode) | 25–40% (camera continuous) |
| Sync loss on device reboot (offline queue) | 0 events lost | Possible (browser storage) |

End of Document 2 — SGS BagScan Native App Specification